UCSB EHS Biological Safety

Medical Waste Self-Inspection Checklist

The purpose of this checklist is to guide laboratory and clinical locations in assuring compliance with California Department of Public Health (CDPH) medical waste regulations. Review the applicable checklist(s) to ensure your area(s) are in compliance.

|  | YES / NO |
| --- | --- |
| Solid Waste: Biohazard Bags - Primary Container |  |
| All biohazard bags used for solid non-sharps waste must be RED in color, labeled with the word “BIOHAZARD” and the universal biohazard symbol (NO orange, clear, or black bags). |  |
| Bags containing biohazardous waste must always be in a labeled, rigid, secondary container with tight-fitting lids that are made of a smooth, cleanable material. |  |
| Biohazard bags are completely intact (e.g., no visible holes or waste penetrating the bag). |  |
| Red biohazard bags lining waste containers must fit appropriately inside the secondary container. Bags should not be too big or too small for the container. |  |
| Serological pipettes, or other non-sharp pointed objects, which can puncture the biohazard bags should be double bagged. |  |
| Biohazard bags are not used to cover equipment, or to store anything other than medical waste. |  |
| Biohazard bags are tied properly to prevent leakage or expulsion of contents (i.e., secured with an over-hand knot, gooseneck, zip tie, a twist tie, or nonporous tape). |  |
| Only solid wastes are discarded into the solid secondary waste containers to prevent liquid leakage. Liquids should be removed and decontaminated separately. |  |
| No sharps, pipettes, pipette tips or other biohazardous waste should be on the floors or bench tops. |  |
| Biohazard bags used within the facility must be certified by the manufacturer as having passed the impact resistance (ASTM D1709) tests. |  |
| For offsite transport by a medical waste vendor, biohazard bags must be marked and certified by the manufacturer as having passed both the tear resistance (“ASTM D1922”) and impact resistance (“ASTM D1709”) tests. |  |
|  | YES / NO |
| Solid Waste: Biohazard Bins – Secondary Container |  |
| All biohazardous waste bags must be enclosed within a solid waste container that is: rigid, puncture resistant, leak resistant, composed of a smooth cleanable material, properly labeled with biohazard stickers on all visible sides, and tightly lidded. |  |
| Labels on biohazardous waste containers must display the universal biohazard symbol, labeled “BIOHAZARD”, be intact, nonporous, legible, and visible on the lid and from all lateral sides. |  |
| If a container is labeled biohazard it must be lined with a RED biohazard bag—even if it is empty. |  |
| Tight fitting lids must be secured on all biohazardous waste containers. These lids must be closed shut when you are not actively collecting waste. |  |
| Secondary containers must be intact, in proper working condition (no broken foot pedals) and sanitized after each use. No other items other than biohazard waste bags should be in the container (no absorbent pads, deodorizers). |  |
| No cardboard biohazardous waste containers are used, as these are not leak proof or cleanable. |  |
| No ring stands are used with red biohazard waste bags. |  |
| No items should ever be placed on top of the medical waste containers (e.g. notebook or Kimwipes). |  |
| Biohazard waste bags or containers are not overfilled to allow room for proper closure of the bags (typically <3/4 full). |  |
| Secondary containers which contain bags of Pathology Waste must be labeled to ensure proper segregation from waste containing Biohazards. |  |
| Waste is never removed, compacted, or packed-down before closing the bags. |  |
| RED biohazard bags containing medical waste and stored in secondary containment above 0°C are treated within 7 days from the day the bag is full. |  |
| RED biohazard bags containing medical waste and stored in secondary containment below 0°C are treated within 90 days from the day the bag is full. |  |
| Biohazard bags must be closed with a gooseneck or overhand knot prior to transport. |  |
| All biohazard waste containers must be in a secured location (locked rooms) where only authorized trained personnel are permitted (e.g., no unattended biohazardous waste left in public hallways, autoclave rooms, or deposited outside of an authorized accumulation site). |  |
|  | YES / NO |
| Liquid Biohazard Waste |  |
| Liquid wastes are collected in vessels and decontaminated with a chemical disinfectant with documented effectiveness against the agents in the waste prior to disposal (e.g., final 10% v/v bleach for >10 min). |  |
| Vacuum aspirator traps are equipped with inline HEPA filters to protect the vacuum line. |  |
| Liquid Biohazard Waste is not stored for > 7 days at room temperature. |  |
| Liquid Biohazard Waste is not disposed of in solid medical waste containers. |  |
| For mixed biological and chemical hazardous waste, deactivate the biological materials first and then dispose of the liquid waste via EHS Hazardous Waste. |  |
| Bleach is permitted for sink disposal, but if you are using chemical hazards, including pharmaceuticals, chemotherapy agents or chemical disinfectants other than bleach, liquids are disposed of via EHS Hazardous Waste. |  |
|  | YES / NO |
| All Sharps Waste |  |
| Any waste with acute rigid corners, edges or protuberances capable of cutting or piercing human skin must be discarded in a sharps waste container (e.g. needles, glass pipets, fine wires and broken glass). |  |
| RED biohazard sharps containers are not used for any purpose other than collection of biohazard sharps; no mixed wasted such as liquid, household, chemical, or radioactive material is permitted in a biohazard sharps container. |  |
| Red biohazard sharps containers are single use and unlined |  |
| Re-purposed biohazardous sharps containers are clearly marked as “non-biohazardous”, and all biohazard markings are covered completely. |  |
| Trace Chemotherapy and Pharmaceutical Sharps waste is segregated from biohazardous sharps waste. |  |
| Pharmaceutical wastes are not discarded as Biohazardous sharps, but in a Pharmaceutical Waste Container. |  |
| Sharps containers do not have sharps above the fill line (typically ~2/3 full). |  |
| No items are placed on top of the sharps containers. |  |
| Sharps containers are maintained in an upright position. |  |
| Sharps containers and lids must be completely intact, never cracked or broken (A smaller cracked container must be re-packaged inside a larger sharps container, if necessary). |  |
| When full, biohazardous sharps containers are taped shut, autoclaved and then picked up by EHS Hazardous Waste or brought to the “Autoclaved Sharps Waste” accumulation site in Bio II room 4106, LSB room 2204, or LSB room 4218 where it is picked up without a request. |  |
|  | YES / NO |
| Pathology Waste |  |
| Any recognizable human tissues, infectious animal carcasses or unfixed recognizable animal tissues must be discarded as pathology waste. |  |
| Carcasses and tissues are collected in red biohazard bags in secondary containers labeled with “PATHOLOGY WASTE” or “PATHOLOGY, INCINERATION ONLY”. |  |
| Fixed tissues are disposed of via EHS Chemical Hazardous Waste |  |
| Liquid fixatives or preservatives should be decanted and discarded through Chemical Hazardous waste. |  |
| Any pathology waste (human tissues or infectious animals carcasses/tissues) which are stored frozen (below 0°C) must not be retained for more than 90 days from the start date of accumulation. |  |
| Pathology waste is bagged in red biohazardous waste bags, tied closed, and stored in a -20°C freezer secured in the Animal Resource Center. |  |
| Pathology waste is hand carried to the pickup location used by the licensed commercial medical waste hauler in a rigid and lidded container labeled as “PATHOLOGY WASTE” on the lid and sides so as to be visible from any lateral direction. |  |
| The pathology waste container does not need to be lined if the pathology waste is lined in a red biohazard bag. If the carcass is not already in a biohazard bag then it would be expected that the pathology waste container be lined with a red biohazard bag. |  |
| Infectious animal carcasses may be stored in a dedicated freezer with signage on the freezer Or Infectious animals carcasses may be stored in a shared freezer with signage on the secondary container within the freezer |  |
| Signage on transport and secondary containers needs to read “Pathology ONLY” with the biohazard symbol. |  |
|  | YES / NO |
| Trace Chemotherapy and Pharmaceutical Waste |  |
| Items containing chemotherapy agents which can be scraped or poured must not be disposed as Medical waste; dispose of this waste as Chemical Hazardous Waste. |  |
| Items which have been potentially in contact with Chemotherapy or Pharmaceutical agents must not be disposed as Biohazardous waste. |  |
| Pharmaceutical waste not meeting the definition of a RCRA hazardous waste or a Controlled Substance is disposed of through the EHS Hazardous Waste Program as hazardous chemical waste. |  |
|  | YES / NO |
| Medical Waste Transport |  |
| Prior to transport, bags are secured and closed inside the lab and then transported to the medical waste accumulation site inside labeled, rigid, secondary containers (described above). The transport container lid must be secured shut and the exterior of the container must be decontaminated before leaving the lab. |  |
| All medical waste is transported in a clean, tightly lidded secondary container using a cart or dolly, i.e., never hand-carried or transported on open carts or bins. |  |
| The secondary transport container must be rigid, puncture-proof, leak-resistant, and be labeled with the biohazard symbol and the word “BIOHAZARD” on the lid & all lateral sides. |  |
| Ensure all members of the laboratory are aware of these standard operating procedures. CDPH selects personnel for random quizzing each year. |  |
|  | YES / NO |
| Accumulation Site |  |
| The accumulation area has signage reading “CAUTION - BIOHAZARDOUS WASTE STORAGE AREA – UNAUTHORIZED PERSONS KEEP OUT; CUIDADO – ZONE DE RESIDUOS BIOLOGICOS PELIGROSOS – PROHIBIDA LA ENTRADA A PERSONAS NO AUTORIZADAS,” the word “BIOHAZARD” and the universal biohazard symbol. |  |
| At the accumulation site, bags containing biohazardous waste (but not pathology waste, trace chemo or pharma) are transferred into a container labeled “BIOHAZARD”. |  |
| At the accumulation site, bags containing Pathology waste are transferred to secondary container labeled “PATH” in the -20°C freezer. |  |
| Only Medical Waste (biohazard, biohazard sharps, pharma, pathology, trace chemo) is deposited in the accumulation site. No municipal, chemical/hazardous, radiation or mixed waste is permitted. |  |
| The accumulation site space, floors, and waste containers are clean/sanitary (not soiled). No evidence of spills, leakage or expulsion of material should be evident at the accumulation site. |  |