|  | Walk-in Temperature Controlled Room Life Safety Procedures This procedure shall be posted near the door entrance. | 2/20/2019 |
| --- | --- | --- |

Principal Investigators/Supervisors and those responsible for access to walk-in temperature controlled rooms shall ensure that all personnel understand the potential life-safety hazards associated with these spaces and understand the emergency egress procedures, as defined below.

Personnel entering room shall inform the responsible person that they are entering the walk-in temperature controlled room, and follow life safety procedures as described below. Working alone without a buddy system or check-in schedule1 is not permitted.  In case of an emergency:

Emergency number and location info is posted within the interior door.

Before entering: familiarize yourself with the location of any safety devices present, such as a panic button, emergency release mechanism, firefighter’s axe and lighting.  If present, test the emergency release mechanism and other safety features with your buddy outside the walk-in cooler/freezer to ensure that everything is properly operating.  If present, make sure employee(s) are trained on firefighters axe use.  Promptly report any electrical and mechanical issues to:

Facilities Management (805) 893-8900.

The following precautions shall be used by personnel:

• use a buddy system or a check-in system1

• for check-in system, bring charged cell phone (test signal inside!)
            • if feasible, secure door in open position when inside cooler/freezer

• Wear appropriate clothing to protect from temperature extremes.

Unless suitable ventilation is provided, the risk of low oxygen or hazardous 
  atmospheres is particularly real when;

      - compressed gases or volatile chemicals are stored or used
      - persons work within for any length of time
      - storing or using dry ice, liquid nitrogen, or other cryogenic asphyxiant

Contact EHS: 893-8243 with any questions

1See reverse side regarding Buddy system and Check-in schedule

What is the Buddy System and Check-in schedule to support worker safety?

Being trapped inside a walk-in cooler can quickly become a life-threatening situation.  The hazards present are extreme temperatures and a potentially oxygen-deficient atmosphere.  Individuals exposed to these hazards can quickly succumb to hypothermia and asphyxiation.  Therefore, it is imperative that employees not work alone in these environments.  To address this:

Individuals can employee a buddy system when entering the walk-in. The lab worker brings a coworker along to the walk-in unit to keep watch outside the unit until the task is finished.

The check-in system involves notifying a coworker that you will be entering the walk-in.  In this case, the co-worker does not come along, but stops by on a pre-defined regular basis to confirm that the employee in the unit is OK.  This approach is acceptable if the walk-in unit has a functioning alarm and/or there is adequate cell phone reception in the unit for the employee to call out.