| 1. Pre-Lift Preparation | Incorrect vehicle weight or lift capacity mismatch Damaged or malfunctioning lift equipment | Verify vehicle weight is within lift capacity Inspect lift arms, pads, and controls before use Ensure lift is in good working condition |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2. Identify Proper Lift Points | Improper contact point selection (e.g., tapered or uneven frame surfaces) Inadequate support leading to instability | Use lift safety booklet attached to lift Consult AllData or manufacturer guidance if needed Confirm contact points are flat, stable, and manufacturer-recommended Avoid tapered or irregular frame surfaces |  |  |  |
| 3. Position Lift Arms and Contact Pads | Misalignment of lift arms Uneven load distribution | Align lift arms evenly on both sides Ensure pads are centered and fully contacting flat lift points Adjust arms to achieve balanced load distribution |  |  |  |
| 4. Initial Lift and Stability Verification | Vehicle shift or slip during initial elevation | Raise vehicle slightly (1–3 inches) Perform stability check (gentle push/bounce test) Visually confirm proper contact Lower and reposition if instability is observed |  |  |  |
| 5. Raise Vehicle to Working Height | Loss of stability during elevation Struck-by hazard from shifting vehicle | Maintain safe body positioning during lift Avoid standing directly behind or in movement path Raise vehicle smoothly and evenly Engage mechanical safety locks once elevated |  |  |  |
| 6. Work Under Elevated Vehicle | Vehicle instability Struck-by hazard if shift occurs | Ensure lift is secured on mechanical safety locks Do not rely solely on hydraulic pressure Maintain awareness of vehicle position Periodically verify stability during work |  |  |  |
| 7. Lower Vehicle | Pinch/crush hazards Unexpected movement | Clear personnel and tools from area Disengage locks properly Lower vehicle in a controlled manner |  |  |  |
| Required Training |  | Required PPE Scope-specific PPE |  |  |  |
| Annual and new employee hydraulic lift training / use of reference materials / hazards and controls |  | Refer to shop supervisor |  | Refer to shop supervisor |  |
| I have read and understood the contents of this job hazard analysis: |  |  |  |  |  |
| Name |  |  | Date |  |  |

JHA Template Blank - Revision 1 - University of California at Santa Barbara, Environmental Health & Safety Department